A research project on the intersection of Mission Blvd & Las Palmas Ave is a great choice because it connects a high-speed state highway with a residential/school neighborhood.

Here is a structured Research Plan designed for a high school level, focusing on Level of Service (LOS) and Delay Analysis.

## Project Title: Efficiency and Delay Analysis of the Mission Blvd & Las Palmas Intersection

### 1. The Research Question

"To what extent does the current signal timing at Mission & Las Palmas minimize vehicle delay while maintaining safety for pedestrians?"

### 2. Methodology: Data Collection

You don't need expensive equipment; you can collect professional-grade data with a clipboard, a stopwatch, and a partner.

- Field Observation (The "Turning Movement Count"): * When: Record for 1 hour during the "Peak" (e.g., 7:45 AM – 8:45 AM) and 1 hour during "Off-Peak" (e.g., 10:00 AM – 11:00 AM).

  - What: Tally every car that turns left, goes straight, or turns right for all four "legs" of the intersection.

  - Intervals: Group your data into 15-minute increments. This allows you to calculate the Peak Hour Factor (PHF), which shows how "bursty" the traffic is.

- Delay Measurement:

  - Pick one lane (e.g., Las Palmas heading East).

  - Count the number of cars stopped at the red light.

  - Use a stopwatch to measure the Control Delay: the time from when a car starts braking to when it clears the intersection.

- Signal Timing Log:

  - Sit at the corner and record the duration (in seconds) of the Green, Yellow, and Red lights for each direction. Do this for 3 full cycles to see if the timing is "fixed" or "actuated" (changing based on demand).

### 3. Data Analysis (The "Math" Part)

This is where you turn your raw numbers into a scientific grade.

- Calculate Level of Service (LOS): Use the average delay you measured to "grade" the intersection.

  - LOS A: < 10s delay

  - LOS C: 20–35s delay (Goal for urban areas)

  - LOS F: > 80s delay (Failing)

- Saturation Flow Rate: Calculate how many cars actually pass through per second of green light.

  - Formula: $S = \frac{n}{G}$ (where $n$ is number of cars and $G$ is green time).

- Pedestrian vs. Vehicle Conflict: Count how many times a pedestrian pushes the button and how long they wait. Compare this to the vehicle green time. Is it balanced?

### 4. Proposed Timeline

| Phase | Task | Time Estimate |
| --- | --- | --- |
| Week 1 | Map out the intersection and identify safe spots for observation. | 2 hours |
| Week 2 | Conduct Field Observations (2 hours total). | 1 Day |
| Week 3 | Crunch the numbers (Calculate LOS and PHF). | 3 hours |
| Week 4 | Write the Report/Create the Presentation Board. | 5 hours |

### 5. Essential Tips for Students

- Safety First: Stand well back from the curb. Wear a high-visibility vest if possible so drivers and police know you are a student researcher.

- The "School Factor": Since this is near Mission San Jose High and Ohlone College, note in your project how school drop-off times creates a unique "surge" compared to standard commute traffic.

- Hypothesis Tip: A good hypothesis might be: "I predict Las Palmas Ave experiences a lower Level of Service (higher delay) than Mission Blvd because the signal is timed to prioritize the state highway over local residential traffic."

Would you like me to create a "Tally Sheet" template that you can print out and use during your field observation?