Here is a professional-grade Intersection Tally Sheet and a step-by-step guide on how to use it. You can print this out or recreate it in a notebook for your field research.

## Intersection Tally Sheet: Mission Blvd & Las Palmas

Date: __________ | Time Start: __________ | Weather: __________

### 1. Turning Movement Counts (TMC)

Use tally marks (||||) for every car that passes. Use a new row for every 15-minute interval.

| Time Interval | Leg (Direction) | Left Turn | Straight (Thru) | Right Turn |
| --- | --- | --- | --- | --- |
| 0:00 - 0:15 | Mission Blvd (Northbound) |  |  |  |
|  | Mission Blvd (Southbound) |  |  |  |
|  | Las Palmas (Eastbound) |  |  |  |
|  | Las Palmas (Westbound) |  |  |  |
| 0:15 - 0:30 | (Repeat above...) |  |  |  |

### 2. Signal Timing Log

Watch the light for 3 full cycles. Use a stopwatch to record the duration in seconds.

| Cycle # | Direction | Green Time (s) | Yellow Time (s) | Red Time (s) |
| --- | --- | --- | --- | --- |
| 1 | Mission Blvd (North/South) |  |  |  |
|  | Las Palmas (East/West) |  |  |  |
| 2 | Mission Blvd (North/South) |  |  |  |
|  | Las Palmas (East/West) |  |  |  |

Note: If the green light time changes every cycle, the signal is Actuated (it’s reacting to cars). If it stays exactly the same, it is Pre-timed.

### 3. Pedestrian & Safety Observations

Keep a separate tally for these "qualitative" factors. These often explain why an intersection feels slow.

- Pedestrian Actuations: How many people pushed the walk button? _______

- "Sneakers": How many cars turned left on a yellow/red light? _______

- Queue Spillback: Does the line of cars reach the next block? (Yes / No)

## How to Calculate Your Results

Once you have your tally marks, you can calculate the Peak Hour Factor (PHF). This is a common metric used by the City of Fremont to see if the road is over-congested.

The formula for PHF is:

$$PHF = \frac{V}{4 \times V_{15}}$$

- $V$: Total vehicles counted in the full hour.

- $V_{15}$: The number of vehicles in your busiest 15-minute window.

- Interpretation: A PHF close to 1.0 means traffic is steady. A PHF of 0.7 or lower means traffic comes in giant, overwhelming bursts (typical for school zones).

### Next Steps for your Project

- Safety Gear: Make sure you bring a clipboard and a bright colored shirt/vest.

- Mapping: Use Google Earth to print a "bird's eye view" of the intersection before you go so you can label the lanes.

- Photos: Take a photo of the sensors (the cameras or black circles on the arms above the lights) to include in your "Hardware" section.

Would you like me to show you how to format your final data into a professional-looking chart for your presentation?